Side & Rear Track Installation:

- Center the long side rails between the top metal supports. (The C channels face out)

- Using the supplied bolts, washers and nuts, install the rails in the existing top mounting holes.

- Place the spacers between the C-channel and the top mounting area and install the bolts. (Photo A)

- Install the self - tapping screws at the rear of the C channels.

- Tighten all hardware.

- Locate the small rail and place the track flush on the inside of the rear strut.

- Pop rivet the track to the strut using the provided rivets.

Sleeve & Enclosure Installation:

- Remove the windshield from the golf car.

- Remove the “sleeves” from the enclosure.

- Wrap the sleeve around the frame so that the cutout for the windshield clip is in the proper position.

- Engage the small zipper so it wraps snugly around the frame. The zipper should be positioned on the back of the windshield frame. (Photo B)

- Install rear and side enclosure panels by sliding them into the bottom C-Channels.

- Zip Enclosure to Sleeve.

- Select the desired valance and slide in to the top C-Channel.

Photo A								  Photo B